Meeting with Lars and Alexa ICP_MS data

1) Sample weight for each reps? Are they all the same?

50mg per replications, Concentration are all already converted to µg/kg sample -> in the “sample_summary” excel sheet of the file -> is there a statistical outlier (either MIN or MAX) then consider remove those outliers of that elemental measurement from the data analysis,

Explanation for the outliers (MAX, MIN): the outliers warning happens only when 2 out of 3 replicates are close in the concentration. HOWEVER, sample can be heterogenous (even for replicate, so should not throw out these outliers. Lars recommended to just keep them as it is

2) Explain the [N20] and [H2] following every metal? What are the next batch sample consist of? => Triple quad, isobaric filtering out, different modes, for example Sodium isotope 23 in Helium

(TASK 1) For [H2]/[H2 HMI]: High matrix load (HMI) -> diluting gas put in to the system, to deal with high abundancy element in sodium or magnesium. Make sense to use just one of these mode -> use H2 only instead of H2 HMI. -> TASK 1 DONE! (14th June 2024)

(TASK 2) For titanium or Niobium (Nb) or any metal that use [N2O] to oxidize interferences, which one of two do we need to use -> go to the core excel file -> use the mode that have the highest QC for maximum recovery rate sheet “Validation_summary” -> look at column Recovery [%] -> if between same metal elements -> for each metal just take the best mode out with highest % .. If no standard for QC -> there will be some metals without recovery % -> if not exist Recovery [%]  values => use “U  (k=2)” instead(this is the column of uncertainty measurement) -> the best one to choose are the ones with lowest uncertainty

IN CONCLUSION: If a metal element was analyzed in multiple modes, including N2O then just keep one of the mode that have the highest  Recovery [%] or lowest "U(k=2)"  -> TASK 2 IS DONE!! (18th June 2024)!

(Issue !!!) Update (14th June 2024): There are isotopic of certain metal that was measured (for example  "69  Ga"         "71  Ga"         "72  Ge"         "74  Ge"). ALSO, there are multiple measurements of these metals with different modes (Helium and N20) -> Ask Lars/Alexa how to deal with it?

(TASK 3) Filtering and cleaning once again with TASK 1 and TASK 2 to see if there is a change in the clustering analyses and ML classification result -> report similar or differences in results in the manuscript in Google Drive. => (Update 18th June): no change in clustering. However, the top 10 PCA PC1 and PC2 contributors are skewed towards metal that may appear abundantly in seawater such as Na, Mg or Ca.

Email to ask Lars and Alexa 2 questions (19th June 2024):

Attach figures:
Top ten PCA PC1 and PC2 contributors and

- For the sample prep of Fabric face mask (the one that have absurbly high Ba, Na and Mg which can be a consequence of seawater) => How did they prepare the samples? Any sample cleaning? Can seawater be an effect that drives this sort of pattern that we observed?

- How did they convert the µg/L to µg/kg (between “samples” to “sample_summary” sheet)?

In the collision cell to get rid of interferences  to  collision and reaction cell , [He], [N20]

3) What are the next batch of sample consist of? -> ElementData_USE_Plastics.xlms

4) Writing of the paper ->  Scope of the paper:

Plastic locational

Need all the data ? -> qualitative conclusions , not really quantitative

UPDATE 24th June 2024 – Answer from Lars

Question 1:

- We took the samples as they arrived (except from drying in clean benches at ambient temperature) since adding any specific cleaning procedures would lead to systematic differences between different analyses.

- From Lar’s perspective, it is likely that the high Ba, Na and Mg mass fractions stem from seawater / attachment of particulate matter (some particles showed some degree of aggregation).   should remove these elements form our analysis

Question 2:

We digest 50 – 100 mg and dilute it to 50 mL (example 50 mg of 50,000 mg (density of water approx 1 g/mL)  Conversion factor 1000).